QVIS Technology Solutions

End-to-End Small Business Network Build

Multi-VLAN  ·  Active Directory  ·  DHCP  ·  DNS  ·  Troubleshooting

Portfolio Project

Olumide Tunde

Network Technical Support Certificate · Conestoga College

May 2026

# Executive Summary

QVIS Technology Solutions Inc. is a Kitchener-Waterloo managed-services and cloud consulting firm launching with ten employees and a twelve-month plan to reach twenty. This portfolio walks through the network and identity infrastructure I designed and built for QVIS as if I were the in-house Network Technician on day one.

The deliverable is a complete, reproducible build: a labelled topology, full Cisco IOS configurations for the edge router, Layer 3 core switch, and two access switches, PowerShell scripts for Active Directory, DHCP, and DNS, and six troubleshooting case studies drawn from realistic Tier-1 and Tier-2 incidents. Every design decision is justified against the size and risk profile of the business.

Skills demonstrated: VLSM subnetting, VLAN design and 802.1Q trunking, inter-VLAN routing on a Layer 3 switch, DHCP relay, NAT overload, named extended ACLs for east-west segmentation, SSH and AAA hardening, native-VLAN security, port-security, AD DS forest build, function-based OU design, AGDLP group nesting, DHCP failover, DNS forward and reverse zones, baseline GPOs, and structured OSI-shaped troubleshooting.

## Quick stats

| Item | Value |
| --- | --- |
| Headcount at launch / target | 10 / 20 |
| VLANs | 10 (8 user + Servers + Native blackhole) |
| Layer-3 routing | Performed on Cisco 3650 SVIs |
| Internal IP range | 10.10.0.0/16 (subnetted into /24 user VLANs) |
| AD forest | qvis.local (Windows Server 2022, 2016 functional level) |
| DHCP redundancy | Hot-standby failover DC1 ↔ DC2 |
| East-west policy | Named extended ACLs on every SVI |
| Build platform | Cisco Packet Tracer 8.x |
| Conestoga lab exposure (training only) | Cisco 2811, Juniper EX4100, Palo Alto NGFW |

Palo Alto next-generation firewall is listed as part of my Conestoga training but is not used in this build — I do not currently have access to a Palo Alto lab environment outside the school. Firewall function in this portfolio lives on the Cisco 2911 edge router (ACL + NAT) and the L3 core switch (east-west ACLs).

# 1.  Company, Staffing and Office Layout

## 1.1  Company profile

| Field | Value |
| --- | --- |
| Legal name | QVIS Technology Solutions Inc. |
| Public domain | qvistech.ca |
| Internal AD domain | qvis.local |
| Location | Kitchener-Waterloo, ON |
| Industry | Managed IT services, cloud, SMB cybersecurity |
| Clients | ~40 SMB clients across legal, dental, accounting, manufacturing |
| Compliance pressure | PIPEDA, client SOC 2 expectations, some PHIPA-adjacent dental work |

## 1.2  Phase 1 — 10 launch employees

| Role | Department | VLAN | OU |
| --- | --- | --- | --- |
| Managing Director / CEO | Executive | EXEC-FIN (30) | Users/Executive |
| Operations Manager | Executive | EXEC-FIN (30) | Users/Executive |
| Finance & HR Manager | Finance-HR | EXEC-FIN (30) | Users/Finance-HR |
| Senior Network Engineer | Technical | TECH (50) | Users/Technical/Network |
| Network Technician (me) | Technical | TECH (50) | Users/Technical/Network |
| Senior Cloud / Sysadmin | Technical | TECH (50) | Users/Technical/Cloud-Sysadmin |
| Helpdesk Tier-1 | Support | TECH (50) | Users/Support-Admin |
| Sales Lead | Sales-Marketing | STAFF (40) | Users/Sales-Marketing |
| Sales Rep | Sales-Marketing | STAFF (40) | Users/Sales-Marketing |
| Office Administrator | Support | STAFF (40) | Users/Support-Admin |

## 1.3  Phase 2 — scaling to 20

Each user VLAN is sized as /24 (254 usable hosts). Doubling the headcount is absorbed without re-IP or scope edits. New hires by month-12 plan: CFO, HR Coordinator, Marketing Specialist, two more Sales Reps, Project Manager, Cybersecurity Analyst, Junior Cloud Engineer, Helpdesk T2, and a second Helpdesk T1.

## 1.4  Office floor plan — twelve rooms

The office is a single floor, roughly rectangular. The server room sits at the geographic centre of the floor so horizontal copper to the furthest desk stays under the 90-metre TIA-568 limit. The plan is divided into a north wing (front-of-house) and a south wing (back-of-house). Each wing is served by its own access switch.

| Room | Wing | Purpose | VLAN(s) |
| --- | --- | --- | --- |
| R1 Reception / Lobby | North | Public entrance, visitors | STAFF + Guest Wi-Fi |
| R2 Executive Office | North | CEO + Ops Mgr | EXEC-FIN |
| R3 Finance / HR Office | North | Restricted, badge entry | EXEC-FIN |
| R4 Open Workspace (Sales bullpen) | North | Sales, Marketing, Admin | STAFF |
| R5 Engineering / Tech Workstations | South | Permanent tech desks | TECH |
| R6 NOC | South | Monitoring + on-call | TECH |
| R7 Server Room / MDF | Centre | Locked, badge entry, climate | SERVERS + MGMT |
| R8 Large Conference Room | North | Client meetings, all-hands | STAFF + VOICE |
| R9 Small Huddle Room | North | 1:1s, interviews | STAFF + VOICE |
| R10 Training Lab | South | Client training, internal labs | LAB (isolated) |
| R11 Break Room / Kitchen | North | Kitchen, IoT (TV, fridge) | Guest only |
| R12 Storage / Receiving | South | Spare hardware, shipping | TECH (1 drop) |

# 2.  Network Design

## 2.1  Topology

Internet enters via R1 (Cisco 2911). R1 hands off a routed /30 to CORE-SW1 (Cisco 3650 Layer 3). CORE-SW1 hosts the SVI gateway for every VLAN and performs inter-VLAN routing in hardware. Two 802.1Q trunks descend to the north-wing and south-wing access switches. Domain controllers and the file server sit on access ports directly on CORE-SW1 inside the server room.

## 2.2  VLAN plan

| VLAN | Name | Subnet | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 10 | MGMT | 10.10.10.0/24 | 10.10.10.1 | Network device management |
| 20 | SERVERS | 10.10.20.0/24 | 10.10.20.1 | DC1, DC2, FS1 |
| 30 | EXEC-FIN | 10.10.30.0/24 | 10.10.30.1 | Executive + Finance + HR |
| 40 | STAFF | 10.10.40.0/24 | 10.10.40.1 | Sales, Marketing, Admin |
| 50 | TECH | 10.10.50.0/24 | 10.10.50.1 | IT / Network / Cloud / Support |
| 60 | LAB | 10.10.60.0/24 | 10.10.60.1 | Training lab — isolated |
| 70 | VOICE | 10.10.70.0/24 | 10.10.70.1 | VoIP phones (aux VLAN) |
| 80 | PRINT | 10.10.80.0/24 | 10.10.80.1 | Printers and MFPs |
| 90 | GUEST | 10.10.90.0/24 | 10.10.90.1 | Guest Wi-Fi — internet only |
| 99 | BLACKHOLE | — | — | Native VLAN on every trunk, no SVI |

## 2.3  East-west policy matrix

Named extended ACLs are applied inbound on each SVI at the core switch. The cell shows whether the row VLAN may reach the column VLAN.

| From → To | EXEC-FIN | STAFF | TECH | LAB | SERVERS | PRINT | GUEST | Internet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXEC-FIN | ✓ | ✗ | ✗ | ✗ | AD/SMB/print | ✓ | ✗ | ✓ |
| STAFF | ✗ | ✓ | ✗ | ✗ | AD/SMB/print | ✓ | ✗ | ✓ |
| TECH | mgmt | mgmt | ✓ | ✓ | ✓ | ✓ | ✗ | ✓ |
| LAB | ✗ | ✗ | ✗ | ✓ | ✗ | ✗ | ✗ | ✓ |
| GUEST | ✗ | ✗ | ✗ | ✗ | ✗ | ✗ | ✓ | ✓ |

Three points worth calling out. TECH can reach everywhere because the IT team supports every other VLAN; that power is balanced by tight admin logging and separate daily-driver vs domain-admin accounts. LAB is one-way isolated — it can reach the internet for training downloads, but nothing on the corporate LAN can reach in. GUEST drops onto a single ACL pattern: deny RFC1918, permit anything else.

## 2.4  Sample IOS — Layer 3 core SVI and ACL

! ---- EXEC-FIN gateway with helper-addresses and inbound policy ---- interface Vlan30 description EXEC-FIN gateway ip address 10.10.30.1 255.255.255.0 ip helper-address 10.10.20.10 ip helper-address 10.10.20.11 ip access-group ACL-EXEC-FIN-IN in no shutdown ! ! ---- The east-west policy that the SVI applies ---- ip access-list extended ACL-EXEC-FIN-IN permit udp 10.10.30.0 0.0.0.255 10.10.20.0 0.0.0.255 eq 53 permit udp 10.10.30.0 0.0.0.255 10.10.20.0 0.0.0.255 eq 67 permit tcp 10.10.30.0 0.0.0.255 10.10.20.0 0.0.0.255 eq 445 permit tcp 10.10.30.0 0.0.0.255 10.10.80.0 0.0.0.255 eq 9100 deny ip 10.10.30.0 0.0.0.255 10.10.0.0 0.0.255.255 log permit ip 10.10.30.0 0.0.0.255 any

Full configurations for R1, CORE-SW1, ACC-SW1 and ACC-SW2 are in the /configs folder beside this document.

# 3.  Active Directory, DHCP, DNS

## 3.1  Identity strategy

| Decision | Choice | Why |
| --- | --- | --- |
| Forest | Single forest, single domain qvis.local | 20 users does not justify multi-domain |
| Domain controllers | DC1 + DC2 | Redundancy; DC2 also hosts NPS/RADIUS |
| Functional level | Win 2016 forest + domain | Future-proof, modern features |
| FSMO roles | All on DC1, runbook to seize on DC2 | Documented but not exercised at launch |
| UPN suffix | user@qvis.local internal | Matches external mail pattern |
| Time | PDC emulator points to time.nist.gov | Domain time hygiene = working Kerberos |

## 3.2  OU design — function-first, not department-first

Function-shaped OUs let GPOs target by behaviour (lock screen, USB block) rather than department, and let computers move with users without re-targeting policy. AGDLP applies throughout: Accounts → Global groups (role) → Domain Local groups (resource permission) → Permission on the resource.

qvis.local └── OU = QVIS ├── OU = _Disabled ← offboarded users / decommissioned PCs ├── OU = ServiceAccounts ├── OU = Users │ ├── OU = Executive │ ├── OU = Finance-HR │ ├── OU = Sales-Marketing │ ├── OU = Technical │ │ ├── OU = Network │ │ └── OU = Cloud-Sysadmin │ └── OU = Support-Admin ├── OU = Computers │ ├── OU = Workstations (mirrors departments) │ ├── OU = Laptops │ └── OU = Servers (FileServers, Application) └── OU = Groups ├── OU = Security │ ├── OU = Role-Based (G_Finance_Users, G_Network_Engineers, ...) │ ├── OU = Resource (DL_Share_Finance_RW, ...) │ └── OU = Privileged (G_NetAdmins, G_Helpdesk_T1, ...) └── OU = Distribution

## 3.3  DHCP

DC1 is primary, DC2 is failover partner in hot-standby mode with a 5% reserve and one-hour MCLT. The L3 core forwards DHCP from each VLAN to both DCs via ip helper-address. Lease durations are eight days for fixed VLANs and four hours for guest and lab.

| Scope | Range | Lease | Notes |
| --- | --- | --- | --- |
| QVIS-EXEC-FIN | 10.10.30.20 – 10.10.30.199 | 8d | Reserved .200+ |
| QVIS-STAFF | 10.10.40.20 – 10.10.40.199 | 8d | — |
| QVIS-TECH | 10.10.50.20 – 10.10.50.199 | 8d | — |
| QVIS-LAB | 10.10.60.50 – 10.10.60.150 | 4h | DNS handed out is DC1 only |
| QVIS-VOICE | 10.10.70.20 – 10.10.70.199 | 8d | Option 150 → TFTP for phones |
| QVIS-PRINT | 10.10.80.20 – 10.10.80.49 | 8d | Reservations only |
| QVIS-GUEST | 10.10.90.20 – 10.10.90.254 | 4h | Public DNS 1.1.1.1/9.9.9.9 |

## 3.4  DNS

qvis.local is AD-integrated with secure dynamic update only. A single reverse zone covers 10.10.0.0/16. Forwarders: 1.1.1.1 and 9.9.9.9 with root hints disabled (fail closed). Scavenging is on with seven-day no-refresh and seven-day refresh windows.

## 3.5  Baseline GPOs

| GPO | Linked to | Effect |
| --- | --- | --- |
| GPO_Baseline_AllUsers | OU=Users | 12-char password complexity, screen lock 10 min, lockout 5/15 |
| GPO_Baseline_AllComputers | OU=Computers | Firewall on, BitLocker required, SmartScreen on, Windows Update |
| GPO_FinanceHR_USB_Block | OU=Users/Finance-HR + Executive | USBSTOR read/write denied |
| GPO_Mapped_Drives | OU=Users | M: \\FS1\Shared, U: \\FS1\Users\%username% + item-level by dept |
| GPO_Helpdesk_LocalAdmin | OU=Computers/Workstations | LAPS deployed, G_Helpdesk_T1 added via restricted groups |
| GPO_Lab_Lockdown | OU=Computers (WMI-filtered) | No corp drives, lab wallpaper, auto-logoff 6 pm |

# 4.  Packet Tracer Build Sequence

Full step-by-step in 04_packet_tracer_build_guide.md. The summary order, which matters: drop devices, cable backbone (R1↔CORE, CORE↔ACC, server access ports), then paste configs in order R1 → CORE-SW1 → ACC-SW1 → ACC-SW2. Verify the backbone with show ip route and a /30 ping before plugging any end devices in. Configure DC1 services (DNS, DHCP) on the Packet Tracer server, then bring up a PC per VLAN and request DHCP. The four pings that prove the network works:

- From R1: ping 10.10.0.2 (CORE) — proves the routed /30 is up

- From CORE: ping 10.10.20.10 (DC1) — proves SVI 20 and server access port

- From any PC: ping its gateway — proves access VLAN + trunk

- From any PC: ping 8.8.8.8 — proves NAT on R1 and default route

Anything that does not work in Packet Tracer (DHCP failover, RADIUS, real Kerberos) is documented in the production runbook in /configs so a paste-and-go deployment to real hardware works.

# 5.  Troubleshooting Case Studies

Six realistic incidents that demonstrate structured triage rather than reboot-and-pray. Every case is treated with the same five-step structure: symptom, triage questions, diagnostic plan up the OSI stack, root cause, fix, prevention.

## Case 1 — "The new sales rep's PC has no network"

### Symptom

Liam, a new Sales Rep, is plugged into R4 (Sales bullpen). His PC shows "No Internet". The phone on the same cable is also dead.

### Triage and diagnostic plan

- Layer 1: link light on the switch port and on the NIC

- Layer 2: show interfaces status on the switch — VLAN as expected? Port up/up?

- Layer 2: show interfaces status err-disabled

- Layer 3: ipconfig /all on the PC, ping the gateway

### Root cause

show interfaces status err-disabled returned Fa0/11 err-disabled psecure-violation. The rep had plugged in a personal USB-NIC; with the phone (MAC #1) and the PC (MAC #2) already on the port, the dongle (MAC #3) tripped the port-security maximum 2 rule and the violation action shutdown closed the port.

### Fix

ACC-SW1# configure terminal interface FastEthernet0/11 shutdown no shutdown end write memory ! optional: clear port-security sticky interface Fa0/11

### Prevention

Port-security did its job. Add a clear line to the new-hire onboarding pack: corporate Wi-Fi only, no rogue USB-NICs. If tickets like this become frequent, consider switching the violation action from shutdown to restrict.

## Case 2 — "All of Sales lost the file server at 11 a.m."

### Symptom

Every Sales PC has internet but cannot reach \\fs1\sales or \\fs1\public. EXEC-FIN and TECH report no problem.

### Diagnostic plan

- From Sales PC: ping 10.10.40.1 (gateway) OK, ping 10.10.20.12 (FS1) fails

- From CORE-SW1: ping 10.10.40.20 OK and ping 10.10.20.12 OK — so the core can reach both

- Means the SVI policy is blocking Sales → SERVERS, not L2 or L3 reachability

- show access-lists ACL-STAFF-IN — look for hit count climbing on a deny line

### Root cause

The Senior Network Engineer was hardening ACL-STAFF-IN and pasted the deny RFC1918 catch-all before the SMB permit lines. ACLs are top-down — first match wins, so Sales → SERVERS matched the deny first.

### Fix

CORE-SW1(config)# ip access-list extended ACL-STAFF-IN CORE-SW1(config-ext-nacl)# no 60 CORE-SW1(config-ext-nacl)# 200 deny ip 10.10.40.0 0.0.0.255 10.10.30.0 0.0.0.255 log ! show access-lists ACL-STAFF-IN to verify ordering

### Prevention

ACL changes go through peer review and a staging device first. Always use explicit sequence numbers. Alert if hit counts on any deny+log line start climbing unexpectedly.

## Case 3 — "Conference room Wi-Fi is unusable during client meetings"

### Symptom

In R8 with twelve or more attendees, laptops drop off Wi-Fi every 30–60 seconds. Same laptops work fine at their desks.

### Diagnostic plan

- L1/RF: WiFi Analyzer walk-through — signal level and 2.4 GHz overlap

- L2: confirm laptops on QVIS-Corp SSID and correct VLAN

- AP: client count and channel utilisation on AP2

### Root cause

AP2 (Bullpen) is the only corporate AP in range of R8. With 12 laptops plus 6 phones, AP2 2.4 GHz airtime is at ~85%. A neighbour tenant's AP overlaps on channel 6, and the conference TV is broadcasting its own SSID for casting on the same channel.

### Fix and prevention

Short term: disable the conference TV's broadcast SSID, force corp clients to 5 GHz with band-steering on. Long term: add a dedicated AP4 in R8, anchored at channel 36 on 5 GHz and channel 1 on 2.4 GHz. Rule of thumb: one AP per 20 active devices or per 1,500 sq ft, whichever is smaller. Wireless surveys belong in every office layout change.

## Case 4 — "I created the user but she can't log in"

### Symptom

Helpdesk T1 created nadia.hassan in AD. Her account is visible in ADUC. Nadia gets "The trust relationship between this workstation and the primary domain failed" on her new laptop.

### Diagnostic plan

- Test-ComputerSecureChannel on the laptop (safe mode w/ networking, local admin) returns False

- Get-ADComputer NADIA-LT01 shows object exists but password is mismatched

### Root cause

The laptop was re-imaged using the same hostname but the old AD computer object was never removed. The local machine account password no longer matches the AD computer object — the secure channel is broken.

### Fix

# On laptop, local admin: Test-ComputerSecureChannel -Repair -Credential (Get-Credential) # Provide a Domain Admin credential (or use LAPS to elevate). Reboot.

### Prevention

The imaging template must run Remove-ADComputer against the old name as part of the destroy step, or generate a fresh hostname every re-image. Deploy LAPS so the helpdesk doesn't need to type Domain Admin creds. Document the runbook and require T1 sign-off on each reimage.

## Case 5 — "DNS works for everyone except the Training Lab"

### Symptom

Lab PCs on VLAN 60 get DHCP, can ping 8.8.8.8 directly, but nslookup google.com returns timeout.

### Diagnostic plan

- nslookup google.com 10.10.20.10 — timeout

- nslookup google.com 1.1.1.1 — answers

- ping 10.10.20.10 — timeout (UDP/53 dropped)

- show access-lists ACL-LAB-IN — confirm permit list

### Root cause

ACL-LAB-IN only permits DNS to host 10.10.20.10. The lab DHCP scope was handing out both DC1 and DC2 as DNS servers. When clients tried DC2, the ACL dropped the packet.

### Fix and prevention

Preferred fix: tighten DHCP to hand out only DC1 in the lab scope. Alternative: widen the ACL to permit DNS to DC2 as well. Pick the tighter option — the lab should not become more reachable into the corporate VLAN than strictly required. Going forward, ACL and DHCP changes share one change document, and a synthetic monitor on the lab VLAN hits both DCs hourly.

## Case 6 — "Finance can't print to their own MFP"

### Symptom

Priya and Adaeze print payroll to PRN3 in the Finance corridor. Jobs spool, never print. Printing to PRN1 in the bullpen works — which is wrong on its own (payroll going past Sales).

### Diagnostic plan

- ping 10.10.80.12 from Finance PC — fails

- ping 10.10.80.12 from CORE-SW1 — succeeds

- show access-lists ACL-EXEC-FIN-IN — print permit line has 0 hits

### Root cause

The replacement PRN3 arrived configured with static 10.10.80.112, not the reserved .12. The print server queue points to .112 and works. But Finance PCs received an old GPO that installed a per-PC TCP/IP queue pointing to .12 — which no longer answers.

### Fix and prevention

Factory-reset the new MFP's NIC so DHCP gives it .12; update FS1 queue back to .12. Replace the per-PC TCP/IP printer ports on Finance laptops with a GPO-deployed shared queue \\FS1\PRN3-Finance. Rule going forward: never direct-IP print, always print through the server. Quarterly: walk the FS1 print queues against Get-DhcpServerv4Reservation.

In all six cases the actual fix was small — one config line, one GPO removal, one DHCP option. The value a technician brings is structured triage, not guessing.

# 6.  Repository Contents

The full set of files that accompanies this document:

| Path | Contents |
| --- | --- |
| README.md | Portfolio entry point |
| QVIS_Portfolio_v2.docx | This document |
| 01_company_overview.md | Company, staff, rooms (long form) |
| 02_network_design.md | VLAN, IP plan, ACL matrix (long form) |
| 03_active_directory.md | AD, OU, AGDLP, DHCP, DNS, GPO (long form) |
| 04_packet_tracer_build_guide.md | Step-by-step .pkt construction |
| 05_troubleshooting_case_studies.md | Long-form troubleshooting |
| diagrams/qvis_topology.svg | Labelled topology (vector) |
| diagrams/qvis_topology.png | Same topology rasterised |
| configs/R1_edge_router.txt | Cisco 2911 edge router config |
| configs/CORE_SW1_L3.txt | Cisco 3650 L3 core switch config |
| configs/ACC_SW1_access.txt | Cisco 2960 north-wing access switch |
| configs/ACC_SW2_access.txt | Cisco 2960 south-wing access switch |
| configs/ad-setup.ps1 | AD DS install + forest promotion |
| configs/ou-and-groups.ps1 | OU tree + AGDLP groups |
| configs/users-bulk-import.ps1 | Phase-1 users, phase-2 stubs |
| configs/dhcp-scopes.ps1 | DHCP scopes, reservations, failover |
| configs/dns-zones.ps1 | DNS zones, forwarders, scavenging |

## What I would do next

Three iterations beyond this portfolio that would strengthen the design without changing the core architecture: translate the IOS configs into Junos for the EX4100 platform I have lab time on; add a second L3 core switch and HSRP/VRRP at the SVI gateways once headcount passes 30; and integrate the Wi-Fi authentication path with NPS on DC2 so SSID-to-VLAN binding is per-user via RADIUS attributes instead of per-AP. If a Palo Alto NGFW lab environment becomes available, the natural fourth iteration is to re-express the IOS east-west ACLs as App-ID security policies — but that is gated on hardware access.